Encoder & Decoder

Lab no #06

Fall 2019

Fall 2021

CSE202L Digital logic and computer design

Submitted by: Ashfaq Ahmad

Registration No. : 19PWCSE1795

Class Section: B

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Student Signature: ______________

Submitted to:

Engr. Abdullah hameed

January 13, 2021

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

OBJECTIVES

After completing this experiment you will be able to:

- Design and construct Decoder and Encoder

- Verify their truth tables using logic gates

COMPONENTS REQUIRED

 Two 7410, 3 I/P NAND gate

 Three 7432, 2 I/P OR gate

 7404 hex inverter

THEORY

DECODER:

A Decoder is a multiple input multiple output logic circuit which converts coded input into coded output where input and output codes are different. The input code generally has fewer bits than the output code. Each input code word produces a different output code word i.e there is one to one mapping can be expressed in truth table. In the block diagram of Decoder circuit the encoded information is present as n input producing 2n possible outputs. 2n output values are from 0 through out 2n– 1.

ENCODER:

An Encoder is a digital circuit that perform inverse operation of a Decoder. An Encoder has 2n input lines and n output lines. In Encoder the output lines generates the binary code corresponding to the input value. In octal to binary Encoder it has eight inputs, one for each octal digit and three outputs that generate the corresponding binary code. In Encoder it is assumed that only one input has a value of one at any given time otherwise the circuit is meaningless. It has an ambiguila that when all inputs are zero the outputs are zero. The zero outputs can also be generated when D0 = 1.

PROCEDURE

- Connections are given as per circuit diagram.

- Logical inputs are given as per circuit diagram.

- Observe the output and verify the truth table.

LOGIC DIAGRAM FOR DECODER

Truth table:

| input |  |  | output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E | A | B | D0 | D1 | D2 | D3 |
| 1 | x | x |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 |

Real life picture of Decoder:

Proteus pic:

LOGIC DIAGRAM FOR ENCODER

Proteus pic:

TRUTH TABLE

| input |  |  |  |  |  |  |  | output |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y1 | Y2 | Y3 | Y4 | Y5 | Y6 | Y7 | Y8 | A | B | C |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |

REVIEW QUESTIONS

Question no 1:

Question no 2 and 3:

Question no 4: