Magnitude comparator

Lab no #08

Fall 2019

Fall 2021

CSE202L Digital logic and computer design

Submitted by: Ashfaq Ahmad

Registration No. : 19PWCSE1795

Class Section: B

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Student Signature: ______________

Submitted to:

Engr. Abdullah hameed

January 31, 2021

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Objective:

- Realization of 1-bit comparator using logic gates.

- Realization and implementation of 2-bit comparator using logic gates on breadboard.

- Implementation of 4-bit magnitude comparator on breadboard using IC 7485.

Components Required:

- Breadboard.

- IC Type 7486 Quadruple 2-input XOR gates.

- IC Type 7408 Quadruple 2-input AND gates.

- IC Type 7400 Quadruple 2-input NAND gates.

- IC Type 7410 Triple 3-input NAND gates.

- IC Type 74L85 4-bit magnitude comparator.

- Switches for inputs and

- LED displays for outputs.

Theory:

Magnitude comparator is a combinational logic circuit that compares between two binary numbers A and B and determines their relative magnitudes. The output of the circuit is specified by three binary variables whether: A>B, A=B or A<B

1-bit Magnitude Comparator:

A comparator used to compare two 1-bit binary numbers. It has two binary inputs A, B and three binary outputs: greater than, equal and less than relations. Figure 2 below shows the block diagram and truth table of a 1-bit magnitude comparator.

The Boolean functions describing the 1-bit magnitude comparator according to the truth table are:

(A > B) = AB'        (A = B) = A'B' + AB=(A XNOR B)         (A < B ) = A'B

The logic diagram for 1-bit binary comparator implemented by XOR and basic logic gates is shown below in figure 3.

So we conclude that digital comparators actually use Exclusive-NOR gates within their design for comparing their respective pairs of bits.

Note: sir due to insufficient components I performed this lab in a proteus software.

Real life Picture of 1-bit Magnitude comparator:

2-bit Magnitude Comparator:

A comparator used to compare two 2-bit numbers. It has 4 binary inputs (number A: A1A0, number B: B1B0) and 3 binary outputs: greater than, equal and less than relations. Figure 4 below shows the block diagram and truth table of a 2-bit magnitude comparator.

Using key-map, the simplified Boolean function for the outputs A>B, A=B and A<B is shown below:

Based on the simplified Boolean functions for the three outputs A>B, A=B and A<B, the logic diagram of the 2-bit magnitude comparator is shown below:

Real life picture of 2-bit magnitude comparator:

4-bit Magnitude Comparator:

A comparator used to compare two 4-bit words. The two 4-bit numbers are word A: A3A2A1A0, and word B: B3 B2B1B0) So the circuit has 8 inputs and 3 binary outputs: A>B, A=B and A<B.

Figure 6 shows the block diagram and pin configuration of IC 7485 for 4-bit magnitude comparator. Three inputs are available for cascading comparators. This comparator generates an output of 1 at one of three comparison outputs such that:

- If word A is bigger than word B; A>B output (pin 5) is “1”,

- If word A is smaller than word B; A<B output (pin 7) is “1”,

- If word A is equal to word B; A=B output (pin 6) is “1”.

This IC can be used to compare two 4-bit binary words by grounding the cascade inputs A<B (pin 2) and A>B(pin 4) and connecting the cascade input A=B (pin 3) to Vcc.

How does a 4-bit comparator work?

Equality:

Word A equal word B if: A3=B3, A2=B2, A1=B1, A0=B0.

Inequality:

- If A3 = 1 and B3 = 0, then A is greater than B (A>B). Or

- If A3 and B3 are equal, and if A2 = 1 and B2 = 0, then A > B. Or

- If A3 and B3 are equal & A2 and B2 are equal, and if A1 = 1, and B1 = 0, then A>B. Or

- If A3 and B3 are equal, A2 and B2 are equal and A1 and B1 are equal, and if A0 = 1 and B0 = 0,then A > B.

- If A3 = 0 and B3 = 1, then A is less than B (A<B). Or

- If A3 and B3 are equal, and if A2 = 0 and B2 = 1, then A < B. Or

- If A3 and B3 are equal & A2 and B2 are equal, and if A1 = 0, and B1 = 1, then A<B. Or

- If A3 and B3 are equal, A2 and B2 are equal and A1 and B1 are equal, and if A0 = 0 and B0 = 1,then A < B.

Part A: Lab Tasks

Procedure:

- Check all the components for their working.

- Insert the appropriate ICs into the IC base.

- Make connections as shown in the circuit diagram in figure 5.

- Verify the Truth Table and observe the outputs.

- Repeat the same steps but for the circuit diagram in figure 6 and apply

inputs in the following table.

Real life pic:

Record the outputs for the given values of A and B

| A | B | Output |
| --- | --- | --- |
| 1001 | 0110 | A>B |
| 1100 | 1110 | A<B |
| 0011 | 0101 | A<B |
| 0101 | 0101 | A=B |

Conclusions: Magnitude comparator is studied.

Part B: Post Lab Task (Bonus, worth 5%):

- Design an 8-bit comparator using two chips of IC 7485. The connections are given below.

- Verify the given truth table

Truth table verified.

THE END